aFRR Standard Product (Energy Market)

Unofficial translation

| Parameter | Condition |
| --- | --- |
| Mode of activation | Automatic The setpoint is continuously sent to the reserve unit via the System Operator’s SCADA. Activation takes place in steps of 0.1 MW. |
| Full activation time (FAT) | 5 minutes |
| Minimum quantity | 1 MW |
| Bid granularity | 1 MW |
| Maximum quantity | 9,999 MW |
| Minimum duration of delivery period | None |
| Price resolution | 0.01 €/MWh |
| Validity period | 15 minutes; the first validity period of the day starts at 00:00 CET. All validity periods are consecutive and non-overlapping. |
| Price of the bid | Unit: €/MWh |
| Bidding zone | The bidding zone in which the reserve unit or group is located. |
| Divisibility | Fully divisible in steps of 0.1 MW |
| Direction | Upward or downward |
| Links between bids | Linking of aFRR energy bids is not permitted. |

| Preparation period | Up to 30 seconds |
| --- | --- |
| Ramping period | Up to 5 minutes, including the preparation period |
| Deactivation Period | Up to 5 minutes |
| Minimum duration of delivery period | 15 minutes |
| Minimum duration between the end of deactivation and the following activation | None |

aFRR Standard Product (Capacity Market)

| Parameter | Condition |
| --- | --- |
| Direction | Upward or downward |
| Validity period | 1MTU (15 minutes) |
| Minimum quantity | 1 MW |
| Bid granularity | 1 MW |
| Maximum quantity | 9,999 MW, but not more than the prequalified volume |
| Divisibility | Divisible in steps of 1 MW or indivisible, in accordance with the conditions set during prequalification and as specified in the Capacity Market Rules |
| Volume | Unit: MW |
| Price of the bid | Unit: €/MWh |
| Location | Bidding zone |
| Availability | When submitting bids to the capacity market, technical constraints may be defined such as divisibility, maximum consecutive operating time, and minimum pause between two accepted capacity offers, in accordance with the Capacity Market Rules. aFRR energy bids must be fully available and ready for activation during the validity period of the accepted aFRR offer, in at least the volume of the accepted capacity offer. The obligation to submit aFRR offers may be transferred to another prequalified Balancing Service Provider in accordance with the Capacity Market Rules. |

| Links between bids | Linking of bids is permitted as follows: - Block bids Offers with the same direction, volume, and price across consecutive trading periods may be linked into one block bid. In such cases, either all linked bids are accepted or all are rejected. - Linked bids Bids may be linked within the same trading period. Such linked bids shall be accepted or rejected as a whole in the same volume. - Mutually exclusive bids A bid may be submitted as part of a bid curve, from which at most one bid may be accepted. Mutually exclusive linking between aFRR and mFRR capacity bids is not permitted. - Groups of mutually exclusive bids Such groups may include block bids and linked bids. Up to 10 groups of bids may be submitted within one mutually exclusive group. |
| --- | --- |
| Price method | Marginal pricing |